MACHİNE VİSİON HW1

- Analyze the OV7670 Camera datasheet in terms of

- image resolution options it has

- OV7670 outputs only VGA format. ISP scaled VGA image to other resolution that mobile device needed.

- total array of 656 x 488 pixels, with active pixels 640 x 480 in YUV mode

- VGA,CIF  and any size scaling down from CIF to 40x30

- its speed of operation(fps)

- up to 30 frames per second (fps)

- pixel format options

- output supports for a row

- RGB (RGB 4:2:2)

- RGB(5:6:5 5:5:5 4:4:4)

- YUV(4:2:2)

- YCbCr(4:2:2)

- how a pixel is transmitted from the camera

- Transfer 10-bit data to 8-bit After the Analog Processing block, the bayer pattern Raw signal is fed to a 10-bit analog-to-digital (A/D) converter shared by G and BR channels. This A/D converter operates at speeds up to 12 MHz and is fully synchronous to the pixel rate (actual conversion rate is related to the frame rate).

- Analyze the OV2640  Camera datasheet in terms of

a)image resolution options it has

- single-chip UXGA (1632x1232)

- Supports image sizes: UXGA, SXGA, SVGA, and any size scaling down from SXGA to 40x30

b)its speed of operation(fps)

- UXGA/SXGA 15 fps

- SVGA 30 fps

- CIF 60 fps

c)pixel format options

- Output support for Raw RGB, RGB (RGB565/555),

- GRB422

- YUV (422/420)

- YCbCr (4:2:2) formats

d) how a pixel is transmitted from the camera

- Transfer 10-bit data to 8-bit After the Analog Processing block, the bayer pattern Raw signal is fed to a 10-bit analog-to-digital (A/D) converter shared by G and BR channels. This A/D converter operates at speeds up to 20 MHz and is fully synchronous to the pixel rate (actual conversion rate is related to the frame rate).

3) Analyze the ESP32 CAM board in terms of

a)  the microcontroller it has.

- The smallest802.11b/g/n  WiFi BT SoC Module

- Low power 32 bit CPU can also serve the application processor

b)  its CPU speed.

- Up to 160 MHz clock speed Summary computing power up to 600 DMIPS

c)  the RAM and ROM available on the microcontroller.

- Built in 512 KB SRAM external 4MPSRAM

d)Give a brief explanation of how to program the board via Arduino IDE.

First of all i am using macbook pro if you are using Windows follow same steps which has probably different interface.

First download arduino ide from: https://www.arduino.cc/en/main/software

Than setup and open arduino ide.

Open preferences

Than paste this address to additional board manager URLs: https://dl.espressif.com/dl/package_esp32_index.json

Click ok

Now download the software from board manager

Search esp32 from here and install software

Than check esp32 module

Than open an example and run

4)